Mobile Services and Networks

Grupo de Arquitectura y Redes de Computadores

Introduction to simulation in NS3

3 de febrero de 2020

1. Simulating mobile communications

1.1. Objectives

The main objective of this documents is to guide student in its ﬁrst simulation process through NS3
simulator.

1.2. Prerequisites

Meanwhile redacting this document, the most recent stable release of NS3 simulator is the version

3.30.1. The student is free to use any other version of the simulator. We work with basic examples and
commands so probably the following procedure is also valid for next releases of ns3.

In a similar way, the steps that we will describe later are tested in a PC with Debian GNU/Linux
installed.

1.3. Introduction

The student should follow this procedure

Download the all-in-one package from www.nsnam.org (Direct link)

Unzip the ﬁle ns-allinone-3.30.1.tar.bz2

$tar -xjvf ns-allinone-3.30.1.tar.bz2

Install the ns3 following this full list of dependencies , as a summary, in debian:

• pre-requisites (as root):

apt-get install gcc g++ python python3 python3-dev apt-get install mercurial python-setuptools git

apt-get install qt5-default sqlite sqlite3 libsqlite3-dev
apt-get install gir1.2-goocanvas-2.0 python-gi python-gi-cairo python-pygraphviz p
apt-get install libgtk2.0-0 libgtk2.0-dev

apt-get install libxml2 libxml2-dev

apt-get install doxygen graphviz imagemagick
apt-get install texlive texlive-extra-utils texlive-latex-extra
apt-get install python-sphinx dia

apt-get install gdb valgrind

apt-get install gsl-bin libgsl-dev libgsl23 libgslcblas0

1

1. SIMULATING MOBILE COMMUNICATIONS 2

• go to the directory (ns-allinone-3.30.1) and execute build.py with the following options:

./build.py --enable-test --enable-examples

• you should get something like ’build’ ﬁnished successfully (8m45.341s) with a list of modules

built and a list of modules not built. At this point you could start to work with ns3, however, the most interesting source of information for starting are examples and test. We need to
enable examples and test for use them.

• Execute the veriﬁcation script to check that everything is ok. Inside of ns-3.30.1 directory

execute test.py. You should see something like:

. .

281 of 284 tests passed (281 passed, 3 skipped, 0 failed, 0 crashed, 0 valgrind er

At this point, you are ready for start to execute simulations from examples.

1.4. Simulation ﬂow

In ns-3.30.1/examples/stats you can ﬁnd a good example of a full simulation process including:

a wireless topology deﬁned mainly in wiﬁ-example-sim.cc. The topology deﬁnes two nodes with a

802.11 interface each one. Both nodes communicate each other by mean a wireless channel sending
packets from attached applications (implemented in wiﬁ-example-apps.cc/h). The simulation takes
several arguments from line command. The most important argument for us is the distance apart to
place nodes in meters. This simulation put all results in a sqlite3 database for be analysed.

the shell script wiﬁ-example-db.sh executes the simulated topology changing distance between
wireless nodes. After the execution, the script open the database with the results, summarises the
 results in wiﬁ-default.data ﬁle and executes a gnuplot script which build a graph.

The graph shows the amount of packet lost according to the distance between the nodes (ﬁgure 1.1).

We will use this example extensively (removing, changing and adding source lines of code) so you
should do a backup copy in order to recover the original source code whenever you want.

1.5. Lab 1 deliverable

The student should write a report with the results of the steps redacted in each subsection of this section.
So, for each subsection, a graph with the result obtained should be embedded in the report. The student
also should answer each one of the subsection’s questions.

1.5.1. Changing the script

Starting with the original example stats:

to increment the distances used in each iteration of the simulation to: 25 50 75 100 125 145 147
150 152 155 157 160 162 165 167 170 172 175 177 180 185 190 195 200 210 220 230 240 250
 300 350 400 450 500 600 750 1000

to reduce the number of iterations for each distance from ﬁve to one.

to adjust the x-axis of the gnuplot script to show results of the new distance.

1. SIMULATING MOBILE COMMUNICATIONS 3

WiFi Defaults

100

80

60

40

20

0

0 50 100 150 200

Distance (m)

FIGURA 1.1: Figure 1: Graphic generated by example stats of NS3

1.5.2. Changing the antenna parameters of simulation

Let’s start changing the topology in the physical layer. The next source code correspond graphically
with ﬁgure 1.2. In the lines 1 and 2 a node container with two nodes is created ( 1.2.a). A WiﬁHelper is a
 class for connect all elements in a wiﬁ environment. Lines 5-6, 7 and 8 are devoted to create, conceptually,
 a wiﬁ mac card, a wiﬁ physical equipment (equivalent to an antenna) and a wireless channel respectively
 ( 1.2.b)). Finally, ﬁrst we have to attach the channel to the physical layer 1.2.c in the line 9 and next,
 install in each node, the combination of wiﬁPhy and wiﬁMac (conceptually the wiﬁ interface and the
 antenna) using for that the wiﬁHelper (line 10).

1 NodeContainer nodes;
2 nodes.Create (2);

3 NS_LOG_INFO ("Installing WiFi and Internet stack.");
4 WifiHelper wifi = WifiHelper::Default ();

5 NqosWifiMacHelper wifiMac = NqosWifiMacHelper::Default ();
6 wifiMac.SetType ("ns3::AdhocWifiMac");

7 YansWifiPhyHelper wifiPhy = YansWifiPhyHelper::Default ();
 8 YansWifiChannelHelper wifiChannel = YansWifiChannelHelper::Default ();
 9 wifiPhy.SetChannel (wifiChannel.Create ());

10 NetDeviceContainer nodeDevices = wifi.Install (wifiPhy, wifiMac, nodes);

Our main target in this subsection is to change antenna parameters, so:

which object represents the antenna? Which properties do you should change for increase the range
of a successful communication?

1. SIMULATING MOBILE COMMUNICATIONS 4

NodeContainer

a)

AdhocWiଏMac
 YansWiଏPhy

WiଏMac

wiଏPhy

wiଏChannel (YansWiଏChannel)

b)

YansWiଏPhy

wiଏPhy

c)

NodeContainer

d)

FIGURA 1.2: NS3 objects

1. SIMULATING MOBILE COMMUNICATIONS 5

To study the values of the properties to change in order to establish a successfully communication
of 200, 400 and 800 meters. The student should include a table with parameters changed to
get a successful communication and the proof of the working conﬁguration (include also the
corresponding graphics)

1.5.3. The propagation model

As the reader can observe, the lost of connectivity (100 % of packet loss) is achieved in abrupt way.
For simulating high level protocols is ok since they need keep simple low-level communications issues.
However, This lost of connectivity is not very realistic according to the real wiﬁ communication where
the lost of connectivity is done in a soft way. So, it would be more accurate increasing the rate of packet
lost according to the distance between both nodes.

In NS3 one of the main issues is modelling the radio wave propagation. Researchers has been developing
accurate models for simulate radio propagation models. In [PS12] you can ﬁnd a brief description of the
most used in wireless simulation.

The key problem in a radio propagation model is calculate how distance, fading, etc. affects to the
power of the radio wave. In NS3 this is the main issue of the channel class where you have to add a
propagation loss according to the model you want to use. Coming back to our ﬁrst example, you can
see how in line 8 a default wiﬁ channel is created (variable wiﬁChannel) and is attached to the physical
interface (line 9) with defaults parameters.

1 NodeContainer nodes;
2 nodes.Create (2);

3 NS_LOG_INFO ("Installing WiFi and Internet stack.");
4 WifiHelper wifi = WifiHelper::Default ();

5 NqosWifiMacHelper wifiMac = NqosWifiMacHelper::Default ();
6 wifiMac.SetType ("ns3::AdhocWifiMac");

7 YansWifiPhyHelper wifiPhy = YansWifiPhyHelper::Default ();
 8 YansWifiChannelHelper wifiChannel = YansWifiChannelHelper::Default ();
 9 wifiPhy.SetChannel (wifiChannel.Create ());

10 NetDeviceContainer nodeDevices = wifi.Install (wifiPhy, wifiMac, nodes);

In this section:

Surf in the NS3 documentation and list the different types of loss propagation models implemented.

Test one of the radio propagation models using our example (check AddPropagationLoss method of
 class YansWiﬁChannelHelper) and provide a graphic comparing the results of the YansWiﬁChannel
with default loss propagation model and the radio propagation model chosen by you. Feel you free
 of using several propagation models.

Disable the antenna modiﬁcation done in the last section in order to have defaults settings for the
conﬁgurated loss model. In the same way increase the number of test in the area where rate of packet lost
stats to increase (for example testing every 10 meters between 100 and 200 meters if you observe that
between these two distances the loss of packets start to be different of 0).

Bibliografía

[PS12] Ramesh C. Poonia y Vikram Singh. Performance evaluation of Radio Propagation Model for
 Vehicular ad-hoc networks using vanet mobisim and ns-2. International Journal of Distributed
 and Parallel Systems (IJDPS), 2012.

6